The DZ Company, LLC

710 Grove Ct.

Loveland, CO 80537

970-667-2254

# Saguaro Construction Note #9

04/03/2023

Software Update – Rev A.01.03

This software release has this improvement:

- Additional changes were made to improve audio gain by reducing the amount of compression that is allowed.

To install:

- Save the zip file to a convenient place on a PC (not Saguaro)

- Extract the three files (sw_rx.py, sw_rx_globals.py and stream_sdr_to_audio.py)

- Copy the three files to a flash drive

- Eject the flash drive

- Turn on Saguaro

- Exit the Saguaro program if it is running by clicking “Exit”, then “No”, which takes you to Linux

- Insert the flash drive into a spare USB port on your Saguaro

- Linux will see that you have inserted a flash drive and ask you if you want to open it in File Manager. Click Yes

- In File Manager, navigate to the media you inserted, locate the three files on that drive, highlight all three, right click and select “Copy”

- In File Manager, navigate to home/dzkit/Saguaro. You should see the same three files (sw_rx.py, sw_rx_globals.py and stream_sdr_to_audio.py)

- Rename the existing files by adding _orig to the names, e.g., sw_rx_orig.py

- Paste the new files in that same folder

- Exit the File Manager

- Click the “Eject” symbol on the top menu bar

- When you see the “safe to remove” message, remove the flash drive

- Start the Saguaro program by clicking the raspberry, then Other, then Saguaro

- The program will also continue to start automatically when you turn power on.