The DZ Company, LLC

710 Grove Ct.

Loveland, CO 80537

970-667-2254

# Saguaro Construction Note #17

09/16/24

Software Update – Rev A.01.10

This software release adds these features:

* Adds Aeronautical band and 2M ham band at 108-156MHz including band info

* Adds 440MHz ham band (420-468MHz)

* Adds 1256MHz han band (1240-1312MHz)

* Adds S-meter

* Adds agc attack/decay/decay_delay/decay_threshold

* Adds Squelch for NBFM

* Changed text color to a lighter shade of green for better readability

* Added rev info to title bar and changed description to “Multi-band Receiver”

To install:

- Save the zip file to a convenient place on a PC (not Saguaro)

- Extract the three files (sw_rx.py, sw_rx_globals.py and stream_sdr_to_audio.py)

- Copy the three files to a flash drive

- Eject the flash drive

- Turn on Saguaro

- Exit the Saguaro program if it is running by clicking “Exit”, then “No”, which takes you to Linux

- Insert the flash drive into a spare USB port on your Saguaro

- Linux will see that you have inserted a flash drive and ask you if you want to open it in File Manager. Click Yes

- In File Manager, navigate to the media you inserted, locate the three files on that drive, highlight all three, right click and select “Copy”

- In File Manager, navigate to home/dzkit/Saguaro. You should see the same three files (sw_rx.py, sw_rx_globals.py and stream_sdr_to_audio.py)

- Rename the existing files by adding _orig to the names, e.g., sw_rx_orig.py

- Paste the new files in that same folder

- Exit the File Manager

- Click the “Eject” symbol on the top menu bar

- When you see the “safe to remove” message, remove the flash drive

- Bring up a command prompt

- Execute the following commands to install the enhanced AGC settings:

  - git clone https://github.com/pothosware/SoapySDRPlay ./SoapySDRPlay -b additional-settings

  - cd SoapySDRPlay

  - mkdir build

  - cd build

  - cmake ..

  - make

  - sudo make install

- Start the Saguaro program by clicking the raspberry, then Other, then Saguaro

Note: The program will also continue to start automatically when you turn power on.